Pid低速巡线

#include "stm32f10x.h"

#include "stdint.h"

#include "Delay.h"

#include "Motor.h"

#include "Sensor.h"

static int clamp(int v, int min, int max)

{

if (v > max) return max;

if (v < min) return min;

return v;

}

static float sm_l = 0, sm_r = 0;

static void MotorOut(int l, int r)

{

l = clamp(l, 80, 500);

r = clamp(r, 80, 500);

if (l > 0 && l < 90) l = 90;

if (r > 0 && r < 90) r = 90;

sm_l = sm_l * 0.6f + l * 0.4f;

sm_r = sm_r * 0.6f + r * 0.4f;

Motor1_SetSpeed(0, (int)sm_l);

Motor2_SetSpeed(0, (int)sm_r);

Motor3_SetSpeed(0, (int)sm_r);

Motor4_SetSpeed(0, (int)sm_l);

}

int main(void)

{

Sensor_Init();

Motor_Init();

float Kp = 50.0f, Kd = 6.0f;

float last_err = 0;

int last_dir = 1;

int lost_step = 0;

while (1)

{

/* bX=1 表示检测到黑线（已验证：1=黑线） */

uint8_t b0 = !Sensor0_Get_State();

uint8_t b1 = !Sensor1_Get_State();

uint8_t b2 = !Sensor2_Get_State();

uint8_t b3 = !Sensor3_Get_State();

int black = b0 + b1 + b2 + b3;   /* 黑线传感器数量 */

/* 位置：检测到黑线才累加 */

int pos = 0;

if (b0) pos -= 3;

if (b1) pos -= 1;

if (b2) pos += 1;

if (b3) pos += 3;

/* ==== 丢线 ==== */

if (black == 0)

{

lost_step++;

int turn = (last_dir > 0) ? -120 : 120;

if (lost_step > 20) turn = -turn;

MotorOut(200 + turn, 200 - turn);

Delay_ms(5);

continue;

}

/* ==== 十字 ==== */

lost_step = 0;

if (black == 4)

{

MotorOut(150, 150);

Delay_ms(5);

continue;

}

/* ==== PID巡线 ==== */

{

float error = (float)pos / black;

float d = error - last_err;

last_err = error;

float pid = (Kp * error + Kd * d)*1.0f;

/* 动态速度 */

int speed = 180;

int absp = (pos < 0) ? -pos : pos;

if (absp > 4)      speed = 90;

else if (absp < 2) speed = 220;

/* 已验证方向：left=fast right=slow = right turn */

int left  = speed + (int)pid;

int right = speed - (int)pid;

/* 方向记忆 */

if (pos != 0)

last_dir = (pos > 0) ? 1 : -1;

MotorOut(left, right);

}

Delay_ms(5);

}

}

Pid算法极限

#include "stm32f10x.h"

#include "stdint.h"

#include "Delay.h"

#include "Motor.h"

#include "Sensor.h"

static int clamp(int v, int min, int max)

{

if (v > max) return max;

if (v < min) return min;

return v;

}

static float sm_l = 0, sm_r = 0;

static void MotorOut(int l, int r)

{

l = clamp(l, -650, 650);

r = clamp(r, -650, 650);

sm_l = sm_l * 0.3f + l * 0.7f;

sm_r = sm_r * 0.3f + r * 0.7f;

if (sm_l >= 0)

{

Motor1_SetSpeed(0, (int)sm_l);

Motor4_SetSpeed(0, (int)sm_l);

}

else

{

Motor1_SetSpeed(1, (int)(-sm_l));

Motor4_SetSpeed(1, (int)(-sm_l));

}

if (sm_r >= 0)

{

Motor2_SetSpeed(0, (int)sm_r);

Motor3_SetSpeed(0, (int)sm_r);

}

else

{

Motor2_SetSpeed(1, (int)(-sm_r));

Motor3_SetSpeed(1, (int)(-sm_r));

}

}

int main(void)

{

Sensor_Init();

Motor_Init();

float Kp = 35.0f, Kd = 10.0f;

float last_err = 0;

int last_dir = 1;

int lost_step = 0;

float sm_err = 0;

float sm_speed = 240;

float sm_turn = 0;

uint8_t last_pattern = 0;

uint8_t stable_pattern = 0;

uint8_t same_count = 0;

while (1)

{

uint8_t b0 = !Sensor0_Get_State();

uint8_t b1 = !Sensor1_Get_State();

uint8_t b2 = !Sensor2_Get_State();

uint8_t b3 = !Sensor3_Get_State();

uint8_t pattern = (b0 << 3) | (b1 << 2) | (b2 << 1) | b3;

if (pattern == last_pattern)

{

if (same_count < 5) same_count++;

}

else

{

same_count = 0;

last_pattern = pattern;

}

if (same_count >= 1)

{

stable_pattern = pattern;

}

b0 = (stable_pattern >> 3) & 1;

b1 = (stable_pattern >> 2) & 1;

b2 = (stable_pattern >> 1) & 1;

b3 = stable_pattern & 1;

int black = b0 + b1 + b2 + b3;

int pos = 0;

if (b0) pos -= 3;

if (b1) pos -= 1;

if (b2) pos += 1;

if (b3) pos += 3;

/* ¶ªÏß */

if (black == 0)

{

lost_step++;

if (lost_step < 3)

{

MotorOut((int)sm_l, (int)sm_r);

}

else if (last_dir > 0)

{

MotorOut(360, -160);

}

else

{

MotorOut(-160, 360);

}

Delay_ms(5);

continue;

}

lost_step = 0;

/* PIDÑ²Ïß */

{

float error = 0;

if (black == 4)

{

error = sm_err;

}

else if (black == 1)

{

if (b0)      error = -2.5f;

else if (b1) error = -0.45f;

else if (b2) error =  0.45f;

else if (b3) error =  2.5f;

}

else

{

error = (float)pos / black;

}

sm_err = sm_err * 0.65f + error * 0.35f;

error = sm_err;

float d = error - last_err;

if (d > 0.8f) d = 0.8f;

if (d < -0.8f) d = -0.8f;

last_err = error;

float abs_error = (error > 0) ? error : -error;

float turn_gain = 1.0f + abs_error * 0.5f;

float pid = (Kp * error + Kd * d) * turn_gain;

int target_speed = (int)(280 - abs_error * 45);

target_speed = clamp(target_speed, 150, 280);

int target_turn = (int)pid;

target_turn = clamp(target_turn, -430, 430);

/* ???????,??????? */

sm_speed = sm_speed * 0.85f + target_speed * 0.15f;

sm_turn  = sm_turn  * 0.70f + target_turn  * 0.30f;

int speed = (int)sm_speed;

int turn  = (int)sm_turn;

int left  = speed + turn;

int right = speed - turn;

left  = clamp(left,  20, 650);

right = clamp(right, 20, 650);

if (pos != 0)

last_dir = (pos > 0) ? 1 : -1;

MotorOut(left, right);

}

Delay_ms(5);

}

}

Pid“多传感器 + 状态机 + 曲率控制 + 自适应速度 + 模式识别巡线系统”备份

#include "stm32f10x.h"

#include "stdint.h"

#include "Delay.h"

#include "Motor.h"

#include "Sensor.h"

/******************************************************************************

* ¿Éµ÷²ÎÊýºê¶¨Òå

*

* ÐÞ¸ÄÕâÀïµÄÊýÖµ¿ÉÒÔµ÷ÕûÐ¡³µµÄÑ²ÏßÐÐÎª£¬²»ÐèÒª¸Ä¶¯ÏÂÃæµÄÂß¼­´úÂë¡£

******************************************************************************/

/* PID²ÎÊý */

#define PID_KP                  35.0f    /* ±ÈÀýÏµÊý£¬µ±Ç°Îó²î·Å´ó±¶Êý */

#define PID_KD                  10.0f    /* Î¢·ÖÏµÊý£¬Îó²î±ä»¯ÂÊ·Å´ó±¶Êý */

/* ¶¯Ì¬ËÙ¶È²ÎÊý */

#define SPEED_MAX               680      /* ×î´óËÙ¶È£¨Ö±µÀ£©*/

#define SPEED_MIN               260      /* ×îÐ¡ËÙ¶È£¨¼±Íä£©*/

#define SPEED_ERR_DECAY         22.0f    /* Îó²îÃ¿Ôö¼Ó1.0£¬ËÙ¶È½µµÍÖµ */

#define SPEED_INIT              400.0f   /* ËÙ¶ÈÆ½»¬ÂË²¨³õÊ¼Öµ */

/* ×ªÏòÊä³öÏÞ·ù */

#define TURN_LIMIT              600      /* PID×ªÏòÊä³ö×î´óÖµ */

/* µ¥´«¸ÐÆ÷Ñ¹ÏßÊ±µÄ¹Ì¶¨Îó²îÖµ */

#define ERR_OUTER               2.5f     /* ×îÍâ²à´«¸ÐÆ÷Ñ¹ÏßÎó²î */

#define ERR_INNER               0.45f    /* ÄÚ²à´«¸ÐÆ÷Ñ¹ÏßÎó²î */

/* ·ÇÏßÐÔ×ªÏòÔöÒæ */

#define TURN_GAIN_K             0.9f     /* Æ«²îÔ½´óÔöÒæÔ½´ó£¬´ËÖµ¿ØÖÆÔöÒæÇ¿¶È */

/* Æ½»¬ÂË²¨ÏµÊý */

#define ERR_FILTER_OLD          0.65f    /* Îó²îÆ½»¬¡ª¾ÉÖµÈ¨ÖØ */

#define ERR_FILTER_NEW          0.35f    /* Îó²îÆ½»¬¡ªÐÂÖµÈ¨ÖØ */

#define SPEED_FILTER_OLD        0.75f    /* ËÙ¶ÈÆ½»¬¡ª¾ÉÖµÈ¨ÖØ */

#define SPEED_FILTER_NEW        0.25f    /* ËÙ¶ÈÆ½»¬¡ªÐÂÖµÈ¨ÖØ */

#define TURN_FILTER_OLD         0.55f    /* ×ªÏòÆ½»¬¡ª¾ÉÖµÈ¨ÖØ */

#define TURN_FILTER_NEW         0.45f    /* ×ªÏòÆ½»¬¡ªÐÂÖµÈ¨ÖØ */

#define MOTOR_FILTER_OLD        0.15f    /* µç»úÊä³öÆ½»¬¡ª¾ÉÖµÈ¨ÖØ */

#define MOTOR_FILTER_NEW        0.85f    /* µç»úÊä³öÆ½»¬¡ªÐÂÖµÈ¨ÖØ */

#define CURVE_REVERSE_ERR       1.0f

#define CURVE_REVERSE_POS       1

#define CURVE_INNER_REVERSE    -260

#define CURVE_MIN_TURN_EXTRA    60

#define CURVE_MID_INNER_REVERSE -120     /* ???????????????? */

#define CURVE_MID_TURN_EXTRA    20       /* ????????????? */

/* Pattern bits: b0 b1 b2 b3 -> 1000=left outer, 0001=right outer */

#define PATTERN_LEFT_OUTER      0x08

#define PATTERN_LEFT_HEAVY      0x0C

#define PATTERN_LEFT_INNER      0x04

#define PATTERN_RIGHT_INNER     0x02

#define PATTERN_RIGHT_HEAVY     0x03

#define PATTERN_RIGHT_OUTER     0x01

/* Î¢·ÖÏÞ·ù */

#define D_LIMIT                 0.8f     /* DÏî×î´ó±ä»¯Á¿£¬·À´«¸ÐÆ÷Ìø±ä±©×ß */

/* ´«¸ÐÆ÷È¥¶¶ */

#define SENSOR_STABLE_COUNT     0        /* Á¬ÐøN´ÎÏàÍ¬Ä£Ê½²ÅÓÐÐ§ */

/* ¶ªÏß»Ö¸´ */

#define LOST_HOLD_COUNT         3        /* ¶ªÏßºó±£³ÖÉÏ´ÎÊä³öNÖ¡£¬·ÀÎóÅÐ */

#define LOST_FAST_OUTER         600      /* ¶ªÏßÕÒÏß¡ªÍâ²àÂÖËÙ¶È */

#define LOST_FAST_INNER        -200      /* ¶ªÏßÕÒÏß¡ªÄÚ²àÂÖËÙ¶È£¨¸º=ºóÍË£©*/

/* µç»úÊä³öÏÞ·ù */

#define MOTOR_LIMIT             650      /* µç»úPWM¾ø¶ÔÖµ×î´óÖµ */

/* Ö÷Ñ­»·ÖÜÆÚ */

#define LOOP_DELAY_MS           5        /* Ã¿Ö¡ÑÓÊ±(ms) */

static int clamp(int v, int min, int max)

{

if (v > max) return max;

if (v < min) return min;

return v;

}

static float sm_l = 0, sm_r = 0;

static void MotorOut(int l, int r)

{

l = clamp(l, -MOTOR_LIMIT, MOTOR_LIMIT);

r = clamp(r, -MOTOR_LIMIT, MOTOR_LIMIT);

sm_l = sm_l * MOTOR_FILTER_OLD + l * MOTOR_FILTER_NEW;

sm_r = sm_r * MOTOR_FILTER_OLD + r * MOTOR_FILTER_NEW;

if (sm_l >= 0)

{

Motor1_SetSpeed(0, (int)sm_l);

Motor4_SetSpeed(0, (int)sm_l);

}

else

{

Motor1_SetSpeed(1, (int)(-sm_l));

Motor4_SetSpeed(1, (int)(-sm_l));

}

if (sm_r >= 0)

{

Motor2_SetSpeed(0, (int)sm_r);

Motor3_SetSpeed(0, (int)sm_r);

}

else

{

Motor2_SetSpeed(1, (int)(-sm_r));

Motor3_SetSpeed(1, (int)(-sm_r));

}

}

int main(void)

{

Sensor_Init();

Motor_Init();

float Kp = PID_KP, Kd = PID_KD;

float last_err = 0;

int last_dir = 1;

int lost_step = 0;

float sm_err = 0;

float sm_speed = SPEED_INIT;

float sm_turn = 0;

uint8_t last_pattern = 0;

uint8_t stable_pattern = 0;

uint8_t same_count = 0;

while (1)

{

uint8_t b0 = !Sensor0_Get_State();

uint8_t b1 = !Sensor1_Get_State();

uint8_t b2 = !Sensor2_Get_State();

uint8_t b3 = !Sensor3_Get_State();

uint8_t pattern = (b0 << 3) | (b1 << 2) | (b2 << 1) | b3;

/* ´«¸ÐÆ÷Ä£Ê½È¥¶¶£ºÁ¬ÐøÏàÍ¬²ÅÓÐÐ§ */

if (pattern == last_pattern)

{

if (same_count < 5) same_count++;

}

else

{

same_count = 0;

last_pattern = pattern;

}

if (same_count >= SENSOR_STABLE_COUNT)

{

stable_pattern = pattern;

}

b0 = (stable_pattern >> 3) & 1;

b1 = (stable_pattern >> 2) & 1;

b2 = (stable_pattern >> 1) & 1;

b3 = stable_pattern & 1;

int black = b0 + b1 + b2 + b3;

int pos = 0;

if (b0) pos -= 3;

if (b1) pos -= 1;

if (b2) pos += 1;

if (b3) pos += 3;

/* ===== ¶ªÏß ===== */

if (black == 0)

{

lost_step++;

if (lost_step < LOST_HOLD_COUNT)

{

MotorOut((int)sm_l, (int)sm_r);

}

else if (last_dir > 0)

{

MotorOut(LOST_FAST_OUTER, LOST_FAST_INNER);

}

else

{

MotorOut(LOST_FAST_INNER, LOST_FAST_OUTER);

}

Delay_ms(LOOP_DELAY_MS);

continue;

}

lost_step = 0;

/* ===== PIDÑ²Ïß ===== */

{

float error = 0;

if (black == 4)

{

error = sm_err;

}

else if (black == 1)

{

if (b0)      error = -ERR_OUTER;

else if (b1) error = -ERR_INNER;

else if (b2) error =  ERR_INNER;

else if (b3) error =  ERR_OUTER;

}

else

{

error = (float)pos / black;

}

sm_err = sm_err * ERR_FILTER_OLD + error * ERR_FILTER_NEW;

error = sm_err;

float d = error - last_err;

if (d > D_LIMIT) d = D_LIMIT;

if (d < -D_LIMIT) d = -D_LIMIT;

last_err = error;

float abs_error = (error > 0) ? error : -error;

float turn_gain = 1.0f + abs_error * TURN_GAIN_K;

float pid = (Kp * error + Kd * d) * turn_gain;

int target_speed = (int)(SPEED_MAX - abs_error * SPEED_ERR_DECAY);

target_speed = clamp(target_speed, SPEED_MIN, SPEED_MAX);

int target_turn = (int)pid;

target_turn = clamp(target_turn, -TURN_LIMIT, TURN_LIMIT);

sm_speed = sm_speed * SPEED_FILTER_OLD + target_speed * SPEED_FILTER_NEW;

sm_turn  = sm_turn  * TURN_FILTER_OLD + target_turn  * TURN_FILTER_NEW;

int speed = (int)sm_speed;

int turn  = (int)sm_turn;

int curve_reverse = 0;

switch (stable_pattern)

{

case PATTERN_LEFT_OUTER:  /* 1000 */

case PATTERN_LEFT_HEAVY:  /* 1100 */

{

int force_turn;

curve_reverse = 1;

speed = SPEED_MIN;

force_turn = speed - CURVE_INNER_REVERSE + CURVE_MIN_TURN_EXTRA;

turn = -force_turn;

break;

}

case PATTERN_LEFT_INNER:  /* 0100 */

{

int force_turn;

curve_reverse = 1;

speed = SPEED_MIN;

force_turn = speed - CURVE_MID_INNER_REVERSE + CURVE_MID_TURN_EXTRA;

turn = -force_turn;

break;

}

case PATTERN_RIGHT_INNER: /* 0010 */

{

int force_turn;

curve_reverse = 1;

speed = SPEED_MIN;

force_turn = speed - CURVE_MID_INNER_REVERSE + CURVE_MID_TURN_EXTRA;

turn = force_turn;

break;

}

case PATTERN_RIGHT_HEAVY: /* 0011 */

case PATTERN_RIGHT_OUTER: /* 0001 */

{

int force_turn;

curve_reverse = 1;

speed = SPEED_MIN;

force_turn = speed - CURVE_INNER_REVERSE + CURVE_MIN_TURN_EXTRA;

turn = force_turn;

break;

}

default:

break;

}

int left  = speed + turn;

int right = speed - turn;

if (curve_reverse)

{

left  = clamp(left,  CURVE_INNER_REVERSE, MOTOR_LIMIT);

right = clamp(right, CURVE_INNER_REVERSE, MOTOR_LIMIT);

}

else

{

left  = clamp(left,  20, MOTOR_LIMIT);

right = clamp(right, 20, MOTOR_LIMIT);

}

if (pos != 0)

last_dir = (pos > 0) ? 1 : -1;

MotorOut(left, right);

}

Delay_ms(LOOP_DELAY_MS);

}

}